ALU

ALU is one of the necessary component in a processor. It is responsible for arithmetic, logic and immediate operations like Addition, Subtraction, AND, OR, NOR, Left Shift and Right Shift. The unit receives two inputs srcA and srcB each of 32 bit. One input is from the register file and the other input is from the register file or instruction memory directly. ALUControl is a control signal based on which the corresponding operation to be performed by ALU is selected. The output is sent as an address to Data Memory Unit.

Block Diagram:

ALU Control

ALU

srcA

ALU Result

srcB

Functional Simulation Screenshots:

Operation Table:

| Operand 1 (srcA) | Operand 2 (srcB) | ALU Operation (ALUControl) | ALUResult |
| --- | --- | --- | --- |
| 000000FF | 00000003 | 000 (Add) | 00000102 |
| 000000FF | 00000003 | 001 (Subtract) | 000000FC |
| 000000FF | 00000003 | 010 (AND) | 00000003 |
| 000000FF | 00000003 | 011 (OR) | 000000FF |
| 000000FF | 00000003 | 100 (NOR) | FFFFFF00 |
| 000000FF | 00000003 | 101 (Left Shift) | 000007F8 |
| 000000FF | 00000003 | 110 (Right Shift) | E000001F |

Resource Utilization:

Device Utilization Summary:

Timing Analysis:

|  | Synthesis Stage | Place and Route Stage |
| --- | --- | --- |
| LUT FF Pairs Used | 322 | 306 |
| Fully Used LUT FF Pairs | 0 | 0 |
| Number of bonded IOBs | 99 out of 210 | 99 out of 210 |